Slide Operation

EXTEND

- Park Brake Engaged

- Engine Running

- On 120V or Generator Running (HWH uses power from Batteries)

- Suspension Dumped Switch On

- Auto Level with HWH Jacks

- Driver Seat Forward

- Turn Slide Key On

- Wait Up to 45 Seconds for Seal to be deflated, Yellow light will come on

- Hold “EXTEND” Button until Red Pump Light goes out

- Turn slide key off

RETRACT

- Park Brake Engaged

- Engine Running

- On 120V or Generator Running (HWH uses 12V from Batteries)

- Suspension Dumped Switch On

- Re- Auto Level with HWH Jacks (Slide Has Zero Tolerance)

- Driver Seat Forward

- Turn Slide Key On

- Wait Up to 45 Seconds for Seal to be deflated, Yellow light will come on

- Hold “RETRACT” Button until Red Pump Light goes out

- Turn slide key off

NOTE: Do Not Stop Slide Until Fully Extended or Retracted!

Keep EXTEND/RETRACT Button Depressed during slide movement

NOTE:  Seal Deflation valve to be replaced before operating slide room!!!

NOTE: 12V Thomas Slide seal Air Compressor is disconnected.

Reconnect before operating Slide room!!!!.